Lab - Configure Extended VLANs, VTP and DTP

Topology

- Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2:  Use Dynamic Trunking Protocol (DTP) to Form Trunk Links

Part 3: Configure VLAN Trunking Protocol (VTP)

Part 4: Create extended VLAN

Background / Scenario

In this lab you will configure a switched environment where trunks are negotiated and formed via DTP, and VLAN information is propagated automatically through a VTP domain. You will create an extended VLAN and to add it to the VTP domain.

Scalability and management are two critical considerations when creating a large network. VTP and DTP are protocols that improve management and scalability. Extended VLANs enable better scalability in large environments by extending the number of VLANs that can be configured in a switch. VLAN Trunking Protocol (VTP) allows the switches to automatically communicate VLAN information, improving management and scalability. Dynamic Trunking Protocol (DTP) allows the switches to automatically negotiate and establish trunk links. DTP also improves scalability.

Required Resources

3 Switches (Cisco Catalyst 2960)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the routers.

Cable the network as shown in the topology.

Initialize and reload the network devices as necessary.

Console into the device and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the switches from attempting to translate incorrectly entered commands as though they were host names.

Configure the hostnames according to the topology.

Save the running configuration to the startup configuration file.

Use Dynamic Trunking Protocol (DTP) to Form Trunk Links

While access links transport single VLAN frames, trunk links are links designed to carry frames belonging to multiple VLANs. While trunk links can be manually configured, DTP can be used to allow the switches to negotiate and establish trunk links.

Based on the topology, enable and configure DS1 ports F0/1 and F0/3 as DTP desirable:

DS1(config)# interface range F0/1, F0/3

DS1(config-if-range)# switchport mode dynamic desirable

DS1(config-if-range)#

*Mar  1 00:18:00.821: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

*Mar  1 00:18:00.830: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

*Mar  1 00:18:03.841: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

*Mar  1 00:18:03.858: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to upBased on the fact that the ports above were made DTP desirable, should DS1 ports F0/1 and F0/3 become trunk links?

___________________________________________________________________________________

___________________________________________________________________________________

Verify that AS1 port F0/1 was in fact configured as DTP auto (the default setting). If not, use the commands below to configure AS1 port F0/1 as DTP auto:

AS1# sh interfaces F0/1 switchport

Name: Fa0/1

Switchport: Enabled

Administrative Mode: dynamic auto

Operational Mode: trunk

Administrative Trunking Encapsulation: dot1q

Operational Trunking Encapsulation: dot1q

Negotiation of Trunking: On

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 1 (default)

Administrative Native VLAN tagging: enabled

Voice VLAN: none

Administrative private-vlan host-association: none

Administrative private-vlan mapping: none

Administrative private-vlan trunk native VLAN: none

Administrative private-vlan trunk Native VLAN tagging: enabled

Administrative private-vlan trunk encapsulation: dot1q

Administrative private-vlan trunk normal VLANs: none

Administrative private-vlan trunk associations: none

Administrative private-vlan trunk mappings: none

Operational private-vlan: none

Trunking VLANs Enabled: ALL

Pruning VLANs Enabled: 2-1001

Capture Mode Disabled

Capture VLANs Allowed: ALL

Protected: false

Unknown unicast blocked: disabled

Unknown multicast blocked: disabled

Appliance trust: none

What portion of the output above shows the DTP configuration of AS1 port F0/1?

____________________________________________________________________________________

What portion of the output above shows the current status of AS1 port F0/1?

____________________________________________________________________________________

If AS1 F0/1 was not configured as DTP auto, what commands should be used to do so?

____________________________________________________________________________________

____________________________________________________________________________________

Similarly, verify and configure AS2 port F0/3 as DTP auto if it is not already configured as such:

AS2(config)# interface range F0/3

AS2(config-if-range)# switchport mode dynamic auto

AS2# show interfaces F0/3 switchport

Name: Fa0/3

Switchport: Enabled

Administrative Mode: dynamic auto

Operational Mode: trunk

Administrative Trunking Encapsulation: dot1q

Operational Trunking Encapsulation: dot1q

Negotiation of Trunking: On

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 1 (default)

Administrative Native VLAN tagging: enabled

<output omitted>

Configuring VLAN Trunking Protocol (VTP)

VTP is protocol used to communicate VLAN information among VTP domain participating switches. To configure a new VTP domain, follow the steps below:

Configure switch DS1 as VTP server:

DS1(config)# vtp mode server

DS1(config)#

Device mode already VTP Server for VLANS.

Create the VTP domain by assigning it a name. The VTP domain name is CCNA-LAB.

- Note: VTP domain names are case-sensitive.

DS1(config)# vtp domain CCNA-LAB

Changing VTP domain name from NULL to CCNA-LAB

*Mar  1 01:12:07.498: %SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to CCNA-LAB.

DS1(config)#

Use the command vtp password to assign a password to the VTP domain. VTP domain passwords are optional, but recommended because they increase security.

DS1(config)# vtp password cisco12345

Setting device VTP password to cisco12345

Verify the domain was properly created with show vtp status:

DS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 1

VTP Domain Name                 : CCNA-LAB

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 001e.4914.6980

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Feature VLAN:

--------------

VTP Operating Mode                : Server

Maximum VLANs supported locally   : 1005

Number of existing VLANs          : 5

Configuration Revision            : 0

MD5 digest                        : 0xFE 0x1A 0x4F 0xF2 0xF3 0x21 0x57 0xC5

0x01 0xDC 0x3C 0x4A 0xB1 0xCB 0x4A 0x54

- Based on the output above, what is the revision number of the CCNA-LAB domain? What does that mean?

____________________________________________________________________________________

____________________________________________________________________________________

- What portion of the output above indicates a VTP password has been configured for the domain?

____________________________________________________________________________________

____________________________________________________________________________________

Use DS1, the VTP server, to add five VLANs to domain:

DS1(config)# vlan 10

DS1(config-vlan)# vlan 20

DS1(config-vlan)# vlan 30

DS1(config-vlan)# vlan 40

DS1(config-vlan)# vlan 100

DS1(config-vlan)# end

DS1#

Add the access layer switches AS1 and AS2 to the domain as VTP clients:

- Note: It is important to set a new switch to VTP client before adding it to an existing domain. If the new switch contains any leftover VTP configuration, setting it as VTP client minimizes the risk of the new switch modifying the VLANs already present in the domain.

AS1(config)# vtp mode client

Setting device to VTP Client mode for VLANS.

AS1(config)# vtp domain CCNA-LAB

Changing VTP domain name from NULL to CCNA-LAB

AS1(config)#

*Mar  1 01:36:06.161: %SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to CCNA-LAB.vtp pass

AS1(config)# vtp password cisco12345

Setting device VTP password to cisco12345

AS1(config)# end

Verify that AS1 has learned the VLANs added to the domain by DS1:

AS1# show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/9, Fa0/10

Fa0/11, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

10   VLAN0010                         active

20   VLAN0020                         active

30   VLAN0030                         active

40   VLAN0040                         active

100  VLAN0100                         active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

20   enet  100020     1500  -      -      -        -    -        0      0

30   enet  100030     1500  -      -      -        -    -        0      0

40   enet  100040     1500  -      -      -        -    -        0      0

100  enet  100100     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    srb      0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

Verify that AS2 has learned the VLANs added to the domain by DS1:

AS2# show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/11, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

10   VLAN0010                         active

20   VLAN0020                         active

30   VLAN0030                         active

40   VLAN0040                         active

100  VLAN0100                         active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

20   enet  100020     1500  -      -      -        -    -        0      0

30   enet  100030     1500  -      -      -        -    -        0      0

40   enet  100040     1500  -      -      -        -    -        0      0

100  enet  100100     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    srb      0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

Creating Extended VLANs

Extended VLANs, ranging from 1006 to 4094, are a useful in large networks. Create an extended VLAN on AS1:

AS1(config)# vlan 1010

AS1(config)# end

% Failed to create VLANs 1010

Extended VLAN(s) not allowed in current VTP mode.

%Failed to commit extended VLAN(s) changes.

Was the extended VLAN 1010 created? Why?

____________________________________________________________________________________

Make AS1 VTP mode to transparent:

AS1(config)# vtp mode transparent

Setting device to VTP Transparent mode for VLANS.

AS1(config)# end

Verify AS1 is in VTP transparent mode:

AS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 1

VTP Domain Name                 : CCNA-LAB

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 0025.83e6.9980

Configuration last modified by 0.0.0.0 at 3-1-93 02:39:34

Feature VLAN:

--------------

VTP Operating Mode                : Transparent

Maximum VLANs supported locally   : 255

Number of existing VLANs          : 10

Configuration Revision            : 0

MD5 digest                        : 0x38 0x18 0xBA 0x48 0x7F 0x7B 0x4C 0xBB

0x03 0x52 0x07 0x2B 0x33 0xC1 0xC9 0xE6

Create the extended VLAN 1010 on AS1:

AS1(config)# vlan 1010

AS1(config-vlan)# end

AS1#

Verify the extended VLAN 1010 was created on AS1:

AS1# show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/9, Fa0/10

Fa0/11, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

10   VLAN0010                         active

20   VLAN0020                         active

30   VLAN0030                         active

40   VLAN0040                         active

100  VLAN0100                         active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

1010 VLAN1010                         active

<output omitted>

Was extended VLAN 1010 created on AS1? Explain

- ____________________________________________________________________________________

Is extended VLAN 1010 propagated to DS1 or AS2? Why?

- ____________________________________________________________________________________

- ____________________________________________________________________________________

- ____________________________________________________________________________________

For verification purposes only, attempt to change AS1 from VTP transparent mode to VTP server mode:

AS1(config)# vtp mode server

Device mode cannot be VTP Server for VLANS because extended VLAN(s) exist

AS1(config)#

- As expected, AS1 can not be made a VTP version 1 server while it hosts extended VLANs.

- What is the solution if the network design requires extended VLANs to be added to a VTP domain?

- ____________________________________________________________________________________